Lab 6: Build a Small Network in Packet Tracer — Answers

Lesson 6: Basic Networking and Segmentation

### Experiment 6: Basic Network Design, Connectivity Testing & VLAN Segmentation Report

#### 1. Objective

Design, configure, and validate a basic local area network (LAN) using Cisco Packet Tracer. The lab includes IP addressing, verifying end-to-end connectivity using standard ICMP diagnostics, documenting the topology, and proposing an isolation/segmentation model for Staff and Students.

#### 2. Devices & Physical Topology

- Core Router: R1_HQ (Cisco 2911 / 4331)

- Access Switch: SW1_Floor1 (Cisco Catalyst 2960)

- End Devices: 4 Host PCs (PC-Staff1, PC-Staff2, PC-Student1, PC-Student2) & 1 Server (App-Server1)

Plaintext

R1_HQ (Router) ─── [g0/0/0] ─── SW1_Floor1 (Switch)

├── [Fa0/1] ─── PC-Staff1

├── [Fa0/2] ─── PC-Staff2

├── [Fa0/3] ─── PC-Student1

├── [Fa0/4] ─── PC-Student2

└── [Fa0/10] ── App-Server1

#### 3. Network Addressing & Configuration Table

| Device Name | Interface | IP Address | Subnet Mask | Default Gateway | Device Function |
| --- | --- | --- | --- | --- | --- |
| R1_HQ | GigabitEthernet0/0/0 | 10.10.10.1 | 255.255.255.0 | N/A | Default Gateway for LAN |
| SW1_Floor1 | VLAN 1 (SVI) | 10.10.10.2 | 255.255.255.0 | 10.10.10.1 | Switch Management |
| PC-Staff1 | FastEthernet0 | 10.10.10.11 | 255.255.255.0 | 10.10.10.1 | Staff Workstation |
| PC-Staff2 | FastEthernet0 | 10.10.10.12 | 255.255.255.0 | 10.10.10.1 | Staff Workstation |
| PC-Student1 | FastEthernet0 | 10.10.10.21 | 255.255.255.0 | 10.10.10.1 | Student Workstation |
| PC-Student2 | FastEthernet0 | 10.10.10.22 | 255.255.255.0 | 10.10.10.1 | Student Workstation |
| App-Server1 | FastEthernet0 | 10.10.10.50 | 255.255.255.0 | 10.10.10.1 | Central Academic Server |

#### 4. Device Initial Configurations (Cisco IOS)

##### Router (R1_HQ) Configuration

Plaintext

enable

configure terminal

hostname R1_HQ

interface GigabitEthernet0/0/0

description LAN Gateway Connection

ip address 10.10.10.1 255.255.255.0

no shutdown

exit

do copy running-config startup-config

##### Switch (SW1_Floor1) Configuration

Plaintext

enable

configure terminal

hostname SW1_Floor1

interface vlan 1

description Management Interface

ip address 10.10.10.2 255.255.255.0

no shutdown

exit

ip default-gateway 10.10.10.1

do copy running-config startup-config

#### 5. Connectivity Testing Matrix

| Source Device | Target Command | Target IP | Expected Outcome | Verification Status |
| --- | --- | --- | --- | --- |
| PC-Staff1 | ping 10.10.10.1 | Router Gateway | Reply from 10.10.10.1 | SUCCESS |
| PC-Staff1 | ping 10.10.10.12 | PC-Staff2 | Reply from 10.10.10.12 | SUCCESS |
| PC-Student1 | ping 10.10.10.22 | PC-Student2 | Reply from 10.10.10.22 | SUCCESS |
| PC-Staff1 | ping 10.10.10.50 | App-Server1 | Reply from 10.10.10.50 | SUCCESS |
| PC-Staff2 | ping 10.10.10.2 | Switch SVI | Reply from 10.10.10.2 | SUCCESS |

#### 6. Proposed Network Segmentation Architecture

To enhance security and minimize broadcast domains, the single flat network (10.10.10.0/24) should be divided into distinct Virtual LANs (VLANs) utilizing Inter-VLAN routing (Router-on-a-Stick / SVI) and Access Control Lists (ACLs):

| User Group | VLAN ID | Subnet Assignment | Default Gateway | Security Access Policy |
| --- | --- | --- | --- | --- |
| Staff Network | VLAN 10 | 10.10.10.0/24 | 10.10.10.1 | Unrestricted access to academic systems, administrative DBs, and Internet. |
| Student Network | VLAN 20 | 10.10.20.0/24 | 10.10.20.1 | Access limited to LMS servers and web browsing; blocked from Staff VLAN. |
| DMZ / Servers | VLAN 30 | 10.10.30.0/24 | 10.10.30.1 | Restricted services (HTTP/HTTPS/DNS/DHCP) accessible via explicit port filtering. |

#### 7. Student Questions & Detailed Technical Answers

- 1. Which networking device is responsible for forwarding traffic between distinct IP subnets?

Answer: A Router (Layer 3 device) or a Multilayer/Layer 3 Switch is required to forward traffic between different IP networks and VLANs. In this topology, R1_HQ serves as the default gateway that evaluates network-layer header information (IP addresses) to route packets across subnet boundaries.

- 2. Why is network segmentation essential in modern enterprise and academic networks?

Answer: Network segmentation isolates network segments to mitigate security risks and improve performance. Key benefits include:

  - Containment of Security Threats: Restricts malware execution or lateral movement from compromised endpoints (e.g., student devices) to critical administrative databases.

  - Broadcast Suppression: Reduces unnecessary ARP/broadcast traffic, freeing up switch link bandwidth.

  - Granular Policy Enforcement: Allows administrators to implement Access Control Lists (ACLs) tailored specifically to user roles (e.g., granting staff administrative access while enforcing restrictions on guest or student users).

#### 8. Lab Conclusion

The flat Layer 2 network topology was successfully provisioned using subnet 10.10.10.0/24. All host nodes verified end-to-end IP reachability to R1_HQ and SW1_Floor1 via standard ICMP echo tests. To address security vulnerabilities inherent in a shared flat broadcast domain, a multi-VLAN segmentation strategy (VLANs 10, 20, and 30) was designed to establish logical boundaries between operational staff resources and student access environments.